- EFBF-10* – Barrier Free, wall mounted eye/face wash with stainless steel bowl and wall bracket, ½” U.S. made chrome plated brass stay open ball valve, powder coated cast aluminum flag handle.  Unit shall have (4) polypropylene ‘GS Plus’ spray heads with integral “flip-top” dust covers, filters and 1.8-GPM flow control orifices mounted on a chrome plated brass eyewash assembly.  Unit shall include ANSI compliant sign.

- Performance: Unit complies with ADA requirements for accessibility by handicapped persons. Unit shall be fully factory assembled and hydrostatically tested to meet or exceed ANSI Z358.1 – 2004, and come with a full 2-year warranty.

- Fixture:  Guardian Equipment GBF1724